(Description of works )

REMOVAL : Dismantling & Removal of Windows for Refurbishment/Restoration works including a thorough strip down preparation process, Sash frames will be temporarily boarded up with 18mm OSB.

PREPARATION: Sanding, removal of loose paint, dead putty, removal of any broken or cracked glass. Re-glazed with standard float glass , antique cylinder blown glass, or a Requested  pre-ordered product.

RESTORATION: All vulnerable areas of the sash joinery prone to rot will be inspected for Woodwork rebuilding, Carpentry splicing reinforcement joinery if required.

PAINTING : includes internal & external of oil based weather sealant primers following another  2 coats of the chosen finished colour paint ( oil based )

(Sash frames in situ) Restoration involves Inspection to the common areas of the lower frame. If Restoration is considered, 45°  angle cut splicing is the method of Carpentry rebuilding mostly box side ends & sill. All of the rebuilding Restoration woodwork is done using African sapele hardwood,  All images of works will be photo documented during & after the process of rebuilding. Adhesives , resins, chemicals we use are all certified  & approved products  for conservation building projects.

ASSEMBLY : installation of each sash window will be realigned & squared into its correct position following with the rebalancing of weights to window ratio. (weights supplied if required) Supplied and fitting of new sash cords,  supply and fit full draught proofing systems throughout using a brush carrier pile system bottom & top of each sash window. Our bead timber systems are custom  made using  white deal. The Bead systems are all weather primed prior to installation,  Supply and fitting of new  brass wear throughout, sash fasteners, lifts, rings, locks etc. On the completion all Windows will be thoroughly cleaned inside & out & dry polishing the glass.

Www.historicsashwindows.eu